Highlighted specifications indicate updates from Rev B

| Design Specifications |  |  |
| --- | --- | --- |
| Team Name/Number: 09 |  | Date: 08/26/2025 |
| Customer Requirements - Completed final system in a timely manner. - Cost effective. - Made from durable materials. - Medication delivery system that comprises of three individual devices to deliver medications to the correct patients in hospitals. - Set anatomical dimensions for Patient A and B. - Delivery device must be able to travel from the pill dispensary location to the patient. - Biomechanical interaction with an upper extremity of Patient A to signal the delivery system. - Thermal interaction with Patient B to signal the delivery system at high body temperatures. - The system needs to be maneuverable and agile to navigate the course when delivering medication. - Medication delivery should move and deposit a set number of pills to the correct patient zone. - Efficient in terms of both time and distance traveled. - Visually appealing. - Easy for medical staff to handle and store. |  |  |
| Engineering Specifications - All systems must be fully functional by December 6th, 2025 - Collective system must remain under a $1,000 budget - The collective system must contain 0 components made of wood - The collective system must use 0 adhesion materials (duct tape, glue, etc.) - Mechanical fasteners (screws, nuts, bolts, etc.) must be used to secure 100% of the components - The chassis frame must be able to support up to a 25 lb load - Delivery device must not exceed 1ft x 1ft base area - Wearable device must fit subject A’s dimensions: - Thumb circumference- 2.75 inches, pinky circumference- 1.8 inches, wrist circumference- 7 inches, length between top of thumb and wrist- 5 inches, length between top of pinky and thumb- 5.5 inches Measured the Patient’s Dimensions- David Shunk - Wearable device must fit subject B’s dimensions: - Thumb circumference- 2.25 inches, pinky circumference- 1.6 inches, wrist circumference- 6 inches, length between top of thumb and wrist- 4.5 inches, length between top of pinky and thumb- 5 inches Measured the Patient’s Dimensions- Leah Kern - Delivery device must be able to travel 40 feet in a linear path within a 5% margin of error (2 feet). https://equine.ca.uky.edu/content/what-does-%E2%80%98statistically-significant%E2%80%99-actually-mean - Delivery device must be able to turn 90 degrees to left and right with 1% margin of error. - Delivery device must be able to record the distance traveled while delivering medication using an odometer - Delivery devices must display the total distance traveled while delivering medication - Delivery devices must indicate which patient it is responding to via an LED that operates with 3.3 V and 16 mA https://thepihut.com/blogs/raspberry-pi-tutorials/27968772-turning-on-an-led-with-your-raspberry-pis-gpio-pins - Wearable device must be capable of thermal measurements from 98 to over 102 degrees Fahrenheit in an appropriate/non-invasive way https://www.mayoclinic.org/diseases-conditions/fever/in-depth/fever/art-2005099 - Wearable device must be capable of thermal measurements with an accuracy of 1°F or more https://www.dwyeromega.com/en-us/resources/thermistor#:~:text=Thermistors%20are%20highly%20accurate%20(ranging,not%20change%20significantly%20with%20age. - At 102 degrees Fahrenheit, the wearable device must be triggered to send a 5 GHz Wifi signal to the delivery robot https://www.mayoclinic.org/diseases-conditions/fever/in-depth/fever/art-2005099 - Wearable device must be capable of detecting a non-functional biomechanical movement of a patient's upper extremity that has 1 degree of freedom through sensors or buttons. - Once biomechanical movement is detected, the wearable device must be capable of sending a 5 GHz Wifi signal to the delivery device - The signal between the delivery device and wearable devices should be able to span over 50ft - Delivery device must be able to travel a total of 200ft - The velocity of the delivery device should be under 5mph https://www.sciencedirect.com/science/article/pii/S0921889010000680 - The delivery device must be able to run for 10 minutes. (3 rounds of a 200ft course at an average of 3mph times a factor of safety of 1.5) Developing a mobile robot for transport applications in the hospital domain - ScienceDirect - Delivery device must successfully engage with the docking system 95% of the trials - The docking station must interface with a solo cup of the following dimensions 100% of the time - The cup has a height of 3", outer diameter of 3.5" at the opening, and inner diameter 2.25" at the base - Delivery device must individually dispense 2 correct pills into the proper patient’s cup - Delivery device must have the ability to hold at minimum 20 “pills” of size ⅝ inches in length and ⅜ inches in diameter https://candylandstore.com/shop/jewel-jelly-belly-beans/ - In terms of volume, the delivery device must be able to hold 4.9677 in3 volume for the “pills” - 20 JB * 1 gal930 JB*128 ounces1 gal*1.805 in31 ounce= 4.9677 in3 https://www.vcalc.com/wiki/candies-in-container-calculator - The pill chamber within the delivery device must be able to support 22.6 g which amounts to 20 jelly beans - 1.13 g/JB*20 JB = 22.6 g https://www.jellybelly.com/general-facts - The height of the delivery device must not exceed 29”, so that it can fit under a hospital bed https://www.sondercare.com/learn/hospital-beds/what-is-ideal-height-for-home-hospital-bed/#:~:text=The%20most%20common%20height%20range,%2Dinches%20to%2029%2Dinches. - The weight of the delivery device must less than 50 lbs https://www.bls.gov/ors/factsheet/strength.htm#:~:text=However%2C%20if%20they%20delivered%20slightly,weighing%20less%20than%20one%20pound. - The weight of the wearable device must less than 5.1 lbs https://hf.tc.faa.gov/publications/2005-human-factors-guidance-for-the-use-of-handheld/full_text.pdf - The battery in the delivery device should be able to provide 12V at 10 amps and 5V at 3 amps - The battery in the wearable device should have a maximum voltage of 5V https://oxeltech.de/design-guide-for-battery-safety-in-wearable-electronics/ - The battery in the wearable device should be able to provide 5V at 3 amps - The full system should include 3 microcontrollers, one for the delivery device and one for each wearable device. - The battery in the delivery device should be able to be recharged with a charging time of 1 hour https://fpvfc.org/beginners-guide-to-lipo-batteries#:~:text=The%20safest%20way%20to%20charge,discharged%20battery%20around%203.2v%20). - The microcontrollers must be capable of sending signals via wifi. - The microcontrollers must be capable of being powered by a 5V battery supply - The full system should be able to complete 9 out of 10 consecutive trials successfully - Entire system must be considered aesthetically appealing at a ranking of 3 or higher on the Likert Scale by 3-5 unaffiliated people - The wearable device should not cause skin irritation or be uncomfortable via rating 2 or less on a Likert scale of 1-5 by 3-5 unaffiliated people - The full system should complete the course in under 3 minutes |  |  |